===============================================================

This document is mainly to help you install mouse more properly. You can find the model number right beneath the mouse on the adhesive or sculptured label.

Conventions:

DB-9P: stands for D-suB 9 pin Plug , e.g. Male connector.

DB-9S: stands for D-suB 9 pin Socket, e.g. Female connector.

===============================================================

********************************************************************

Model	:	MUS2S, MUS3S

FCC ID	:	JKGMUS2S01

Date: 9-27-1995

1. General description

This mouse can emulate either the Microsoft mouse (2 buttons MS mode) or the Mouse System mouse (3 buttons PC mode). While turning on the computer without any button pressed , it will act as a Microsoft mouse; otherwise, it will act as a Mouse System mouse. The innovative Mouse Driver can automatically recognize the exact mode without any soft command. And 10 additional resolutions provide you to adjust your favorite moving speed and accuracy.

2. System requirements

(1)	Computer	: Compatible with IBM-PC/XT/AT and PS/2 models.

(2)	Disk			: A 3.5“ floppy disk drive.

(3)	DOS version	: V3.0 and above.

(4)	Memory		: above 256K RAM.

(5)	Mouse port	: Serial mouse port: Com1 to Com4.

(6)	Adapter		: optional DB-25S to DB-9P adapter.

3. Mode settings

This mouse will set into Microsoft mouse mode while turning on the power of computer without any button pressed and hold. And vice versa, it will set into Mouse System mouse mode while turning on the power of computer with any of the three buttons pressed and hold. If you are not sure about what mode you have set , you can run the TRMOUSE.EXE and TRTEST.EXE which contained in the Mouse Driver diskette at DOS prompt to check it out. The exact mode setting will be displayed on the test screen.

4. Installation

(1)	Before connecting this mouse, please turn off your computer first.

(2)	Directly plug the mouse cable to DB-9P serial port on the computer or use an optional DB-25S to DB-9P adapter to attach to a DB-25P serial port.

(3a)	As a Microsoft mouse, simply turn on the computer.

(3b) As a Mouse System mouse, pressed and hold any button of the mouse and then turn on the computer.

(4a)	For DOS environment, run the INSTALL.EXE program from the Mouse Driver diskette to copy all the necessary files to your hard disk. It will modify the Autoexec.bat file as well in order to load the mouse driver automatically when you boot up the computer next time. For more details, please refer to DOS manual .

(4b) 	For Windows 3.X environment, double click Windows Setup in the Main Group and choose either the “Microsoft, or PS/2” or “Mouse Sytems serial” as the System Default Mouse Driver.

(4c)	For Windows 95 environment, simply choose Standard Serial Port Mouse for the exact mode setting.

(5)	Restart or warm-booting computer to take the Driver setting into effect.

5. Supports Com1 and Com2, Com3, Com4 serial ports.

You can choose any of them to install this mouse. And in DOS environment, the Mouse Driver (Trmouse.exe) has a default searching priority from PS/2 first and then Com1 to Com4. As long as the first one response to the mouse driver, the respective port will be considered as the system mouse port. Furthermore, you can still use the following soft command to override the searching priority in case of mixing-up chaos with other serial devices.

Command format:	TRMOUSE /n

Where n starts from 1 to 4 which refers to Com1 to Com4, the serial communication port number where the mouse was installed.

6. Software resolution setting

There are 10 grade of the resolutions in the Mouse Driver that can be changed to fit the accuracy and tracking speed with each application. The default is m=5 for 1:1 resolution, i.e. one count for one dot resolution.

Command format : TRMOUSE /Am 	(where m = 0 ~ 9)

10 resolution settings:

m=0  1/6 of the mouse hardware resolution

m=1  1/5 of the mouse hardware resolution

m=2  1/4 of the mouse hardware resolution

m=3  1/3 of the mouse hardware resolution

m=4  1/2 of the mouse hardware resolution

m=5  1:1 the actual mouse hardware resolution

m=6  2 times of the mouse hardware resolution

m=7  3 times of the mouse hardware resolution

m=8  4 times of the mouse hardware resolution

m=9  5 times of the mouse hardware resolution

7. Help function

Help command provides a brief explanation of mouse commands and options.

Command format : TRMOUSE /?

or : TRMOUSE /H

8. Remove from memory

The Mouse Driver is a Terminated and Stay in Resident program. You can remove it by the following command to release memory . Before doing this , you should first remove other TSRs which loaded after the mouse driver.

Command format: TRMOUSE /R

or : TRMOUSE /OFF

********************************************************************

Model	:	MUS2I, MUS3I

FCC ID	:	JKGMUS2I01

Date: 9-27-1995

1. General description

This mouse can emulate either the Microsoft mouse or the PS/2 mouse. When use it with mini-Din 6P to DB-9P adapter , it will act as a PS/2 mouse; otherwise, it will act as a Microsoft mouse. The innovative Mouse Driver can automatically recognize the exact mode without any soft command. And 10 additional resolutions provide you to adjust your favorite moving speed and accuracy.

2. System requirements

(1)	Computer	: Compatible with IBM-PC/XT/AT and PS/2 models.

(2)	Disk			: A 3.5“ floppy disk drive.

(3)	DOS version	: V3.0 and above.

(4)	Memory		: above 256K RAM.

(5)	Mouse port	:	a.) PS/2 mouse port.

b.) Serial mouse port: Com1 to Com4.

(6)	Adapter		:	a.) mini-Din 6P to DB-9P adapter, for PS/2 mode.

b.) optional DB-25S to DB-9P adapter, for MS mode.

3. Mode settings

This mouse can only act as a PS/2 mouse when the mini-DIN 6P to DB-9P adapter with special wiring is used. In all other cases, it will act as a Microsoft mouse. If you are not sure about what mode you have set , you can run the TRMOUSE.EXE and TRTEST.EXE which contained in the Mouse Driver diskette at DOS prompt to check it out. The exact mode setting will be displayed on the test screen.

4. Installation

(1)	Before connecting this mouse, please turn off your computer first.

(2a)	As a PS/2 mouse, use the mini-DIN 6P to DB-9P adapter attach to mouse cable and plug to PS/2 mouse port on the computer.

(2b)	As a Microsoft mouse, directly plug the mouse cable to DB-9P serial port on the computer or use an optional DB-25S to DB-9P adapter to attach to a DB-25P serial port.

(3)	Turn on the computer.

(4a)	For DOS environment, run the INSTALL.EXE program from the Mouse Driver diskette to copy all the necessary files to your hard disk. It will modify the Autoexec.bat file as well in order to load the mouse driver automatically when you boot up the computer next time. For more details, please refer to DOS manual .

(4b) 	For Windows 3.X environment, double click Windows Setup in the Main Group and choose the “Microsoft, or PS/2” as the System Default Mouse Driver.

(4c)	For Windows 95 environment, simply choose either Standard PS/2 Port Mouse or Standard Serial Port Mouse for the exact mode setting.

(5)	Restart or warm-booting computer to take the Driver setting into effect.

5. Supports PS/2 mouse port and Com1, Com2, Com3, Com4 serial ports.

You can choose any of them to install this mouse. And in DOS environment, the Mouse Driver (Trmouse.exe) has a default searching priority from PS/2 first and then Com1 to Com4. As long as the first one response to the mouse driver, the respective port will be considered as the system mouse port. Furthermore, you can still use the following soft command to override the searching priority in case of mixing-up chaos with other serial devices.

5.1.	PS/2 mouse command

Command format:	TRMOUSE /P

Where the P stands for PS/2 mouse.

5.2.	Microsoft mouse command

Command format:	TRMOUSE /n

Where n starts from 1 to 4 which refers to Com1 to Com4, the serial communication port number where the mouse was installed.

6. Software resolution setting

There are 10 grade of the resolutions in the Mouse Driver that can be changed to fit the accuracy and tracking speed with each application. The default is m=5 for 1:1 resolution, i.e. one count for one dot resolution.

Command format : TRMOUSE /Am 	(where m = 0 ~ 9)

10 resolution settings:

m=0  1/6 of the mouse hardware resolution

m=1  1/5 of the mouse hardware resolution

m=2  1/4 of the mouse hardware resolution

m=3  1/3 of the mouse hardware resolution

m=4  1/2 of the mouse hardware resolution

m=5  1:1 the actual mouse hardware resolution

m=6  2 times of the mouse hardware resolution

m=7  3 times of the mouse hardware resolution

m=8  4 times of the mouse hardware resolution

m=9  5 times of the mouse hardware resolution

7. Help function

Help command provides a brief explanation of mouse commands and options.

Command format : TRMOUSE /?

or : TRMOUSE /H

8. Remove from memory

The Mouse Driver is a Terminated and Stay in Resident program. You can remove it by the following command to release memory . Before doing this , you should first remove other TSRs which loaded after the mouse driver.

Command format: TRMOUSE /R

or : TRMOUSE /OFF

********************************************************************

Model	:	MUS2P, MUS3P

FCC ID	:	JGKMUS2P01

Date: 9-27-1995

1. General description

This mouse is fully compatible with standard PS/2 mouse. The innovative Mouse Driver can automatically recognize the exact mode without any soft command. And 10 additional resolutions provide you to adjust your favorite moving speed and accuracy.

2. System requirements

(1)	Computer	: Compatible with IBM-PC/XT/AT and PS/2 models.

(2)	Disk			: A 3.5“ floppy disk drive.

(3)	DOS version	: V3.0 and above.

(4)	Memory		: above 256K RAM.

(5)	Mouse port	: PS/2 mouse port.

3. Installation

(1)	Before connecting this mouse, please turn off your computer first.

(2)	Plug to PS/2 mouse port on the computer.

(3)	Turn on the computer.

(4a)	For DOS environment, run the INSTALL.EXE program from the Mouse Driver diskette to copy all the necessary files to your hard disk. It will modify the Autoexec.bat file as well in order to load the mouse driver automatically when you boot up the computer next time. For more details, please refer to DOS manual .

(4b) 	For Windows 3.X environment, double click Windows Setup in the Main Group and choose the “Microsoft, or PS/2” as the System Default Mouse Driver.

(5)	Restart or warm-booting computer to take the Driver setting into effect.

4. Software resolution setting

There are 10 grade of the resolutions in the Mouse Driver that can be changed to fit the accuracy and tracking speed with each application. The default is m=5 for 1:1 resolution, i.e. one count for one dot resolution.

Command format : TRMOUSE /Am 	(where m = 0 ~ 9)

10 resolution settings:

m=0  1/6 of the mouse hardware resolution

m=1  1/5 of the mouse hardware resolution

m=2  1/4 of the mouse hardware resolution

m=3  1/3 of the mouse hardware resolution

m=4  1/2 of the mouse hardware resolution

m=5  1:1 the actual mouse hardware resolution

m=6  2 times of the mouse hardware resolution

m=7  3 times of the mouse hardware resolution

m=8  4 times of the mouse hardware resolution

m=9  5 times of the mouse hardware resolution

5. Help function

Help command provides a brief explanation of mouse commands and options.

Command format : TRMOUSE /?

or : TRMOUSE /H

6. Remove from memory

The Mouse Driver is a Terminated and Stay in Resident program. You can remove it by the following command to release memory . Before doing this , you should first remove other TSRs which loaded after the mouse driver.

Command format: TRMOUSE /R

or : TRMOUSE /OFF

FCC COMPLIANCE STATEMENT

This device complies with Part 15 of the FCC Rules Class B.  Operation is subject to the  following two conditions:

(1) this device may not cause harmful interference, and (2) this device must accept any

interference received, including any interference that may cause undesired operation.

FCC WARNING STATEMENT

This equipment has been tested and found to comply with the limits for a Class B digital

device, pursuant to Part 15 of the FCC Rules.  These limits are designed to provide rea-

sonable protection against harmful interference in a residential installation.  This equipment generates, uses and can radiate radio frequency energy and, if not installed or

used in accordance with the instructions may cause harmful interference to radio communications.  However, there is no guarantee that interference will not occur in a particular installation.  If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try and correct the interference by one or more of the following measures:

Reorient or relocate the receiving antenna.

Increase the separation between the equipment and receiver.

Connent the equipment into an outlet on a circuit different from that to which the

receiver is connected.

Consult the dealer or an experienced redio/TV technician for help.

CAUTION:

CHANGES OR MODIFICATIONS NOT EXPRESSLY APPROVED BY THE PARTY RESPONSIBLE FOR COMPLIANCE COULD VOID THE USER‘S AUTHORITY TO OPERATE THE EQUIPMENT.